REQUEST FOR PROPOSAL

WOOD FRAMING - LABOR ONLY

| Project: | Rodriguez Residence |
| --- | --- |
| Location: | Oak Valley Estates, Lot 5, Kanarraville, UT 84742 |
| Owner: | ARJR Kanarraville Holdings, LLC |
| General Contractor: | RAC Inc. |
| Project Size: | 6,993 SF (Main ~4,400 SF, Upper ~1,500 SF, Basement ~1,100 SF) |
| RFP Issue Date: | February 2, 2026 |
| Bids Due: | February 16, 2026 by 5:00 PM MST |

# 1. PROJECT DESCRIPTION

Custom single-family residence with attached ADU, 4-car garage, and underground basement (including gun range). Modern contemporary design with flat/low-slope roofs, multiple levels, and extensive glass. Steel moment frame with wood-framed walls, floors, and roof trusses.

# 2. PLANS & SPECIFICATIONS

The following documents are available for review and shall govern the work:

- Architectural Plans: RAC Development, Sheets A100-A806

- Structural Plans: Rosenberg Engineering, Sheets S1-S8 (Job #14040-25)

- Truss Layouts: By truss manufacturer (to be provided)

- Specifications: Per structural notes on S7-S8

Plans available at: https://drive.google.com/drive/folders/1kS-2awScLDAnGS973xD0D28RxynI49Si

# 3. SCOPE OF WORK

This is a LABOR ONLY bid. All materials furnished by GC.

3.1 INCLUDED IN THIS BID:

- All wood wall framing per structural plans S1-S6

- Floor truss installation (trusses by others)

- Roof truss installation and blocking (trusses by others)

- Wall sheathing: 1" OX Board all levels

- Floor sheathing: 1.125" Advantec T&G

- Roof sheathing: 5/8" CDX per A106 (~9,097 SF)

- All headers, beams (LVL), rim boards per schedule

- Shearwall construction per S4-S5 schedule

- All Simpson connectors, straps, holdowns per plans

- Blocking for cabinets, fixtures, railings

- Fire blocking per code

- Temporary bracing during construction

- Coordination with steel contractor for connections

3.2 EXCLUDED FROM THIS BID:

- All materials (furnished by GC)

- Steel beams and columns (by steel contractor)

- Floor and roof trusses (by truss supplier - install only)

- Concrete and foundation work

- Exterior siding and trim

- Roofing materials and installation

- Windows and doors

# 4. FRAMING SPECIFICATIONS

4.1 WALL FRAMING (per S2, S7-S8):

| Wall Type | Stud Size | Spacing | Grade |
| --- | --- | --- | --- |
| 4" Interior | 2x4 DF | 16" O.C. | #2 |
| 5-1/2" Interior | 2x4 DF S2 | 16" O.C. | #2 |
| 6" Ext/Int | 2x6 DF S2 | 16" O.C. | #2 |
| 8" Walls | 1-5/8"x7-1/2" PSL | 16" O.C. | PSL |
| Living (26' tall) | 1-5/8"x14" LSL | 5" O.C. | LSL |
| Entry (20' tall) | 1-5/8"x6" LSL | 16" O.C. | LSL |

4.2 SHEATHING:

- Wall sheathing: 1" OX Board, all exterior walls

- Floor deck: 1.125" Huber Advantec T&G, glued and screwed

- Roof deck: 5/8" CDX plywood

4.3 SHEARWALLS (per S4-S5):

| Mark | Sheathing | Nailing | Anchors |
| --- | --- | --- | --- |
| SW1 | 7/16" APA rated | 8d @ 6" field / 4" edge | Per plan |
| SW2 | 15/32" APA rated | 8d @ 4" field / 4" edge | Per plan |
| SW3 | 15/32" APA rated | 16d @ 3" O.C. | Per plan |
| SW4 | 15/32" both sides | 16d @ 2-1/2" O.C. staggered | Per plan |

# 5. BID REQUIREMENTS

Proposals shall include the following:

- Lump sum labor price for complete framing scope as defined

- Proposed schedule / duration in working days

- Crew size

- List of any exclusions or clarifications

- Proof of insurance (GL $1M/$2M, WC per state requirements)

- Utah contractor license number

# 6. SCHEDULE

| Milestone | Target Date |
| --- | --- |
| RFP Issued | February 2, 2026 |
| Site Walk (optional) | February 9, 2026 @ 10:00 AM |
| Questions Due | February 12, 2026 |
| Bids Due | February 16, 2026 @ 5:00 PM |
| Award (anticipated) | February 20, 2026 |
| Framing Start | Per project schedule |

# 7. GENERAL CONDITIONS

- Contractor to verify all dimensions in field prior to work

- All work shall comply with 2021 IBC / IRC and local amendments

- Framing inspections required per Iron County Building Department

- Contractor responsible for layout from established benchmarks

- Daily cleanup required; dumpster provided by GC

- Porta-john on site provided by GC

- Payment terms: Progress payments per schedule of values, net 30

- Change orders require written approval prior to work

# 8. SUBMISSION

Submit proposals to:

RAC Inc.

Attn: Craig Engel

1267 N. 1390 W.

St. George, UT 84770

Email: racinc16@gmail.com

Questions: Contact Craig Engel at the email above or (435) XXX-XXXX

RAC Inc. reserves the right to reject any or all bids, waive informalities, and award to the bidder deemed most advantageous to the project.