# Request for Proposal

# Site Camera System — Installation Only

Project: Rodriguez Residence

Location: Oak Valley Estates Lot 5, 646 W Liberty Lane, Kanarraville, UT 84742

Delivery: Lump sum for installation labor per attached scope

Reference: All equipment and design are Owner-furnished. See installer reference documents in this folder (e.g. 3_Starlink_Security_Visual.html) for full scope and sequence.

——————————————————————————————

## 1. Scope of Work — Installation Only

Proposals are for INSTALLATION ONLY. All equipment (cameras, poles, Starlink, solar, battery, brackets, concrete) is Owner-furnished. Contractor performs labor per the attached installer reference documents.

Five (5) poles: North (Starlink + solar + camera), South/East/West (camera only). Layout per site diagram.

Concrete: 18" dia hole, 36" deep per pole; 2 bags 80 lb concrete; plumb; 24 hrs cure before mounting.

North pole: Starlink dish (top), Renogy 100W on LINOVISION bracket (30–35° south), aosu camera (MorningRo clamp, aim south), Starlink battery (5–6 ft). Wire battery before solar input. MC4 panel to battery; Starlink cable to battery.

South/East/West poles: aosu camera at 10–11 ft on MorningRo clamp. South→NORTH, East→WEST, West→EAST. Built-in solar cameras — no pole wiring.

Camera app setup: Power all 4 cameras; aosu app; scan QR; connect to Starlink WiFi (2.4 GHz); confirm live feed. Starlink must be live first.

——————————————————————————————

## 2. Owner-Furnished

All poles, hardware, Starlink (dish, battery), Renogy 100W, LINOVISION bracket, aosu cameras (4), MorningRo clamps, concrete, and materials per installer reference docs.

——————————————————————————————

## 3. Contractor Provides

Labor: layout, excavation, concrete, pole setting, mounting per reference, north-pole wiring, camera app setup and verification. Tools/equipment; state any beyond basic. Confirm Starlink and all 4 cameras live before demobilization.

——————————————————————————————

## 4. Pricing

Lump sum for installation only. Payment terms. Exclusions or assumptions (access, staging, Owner presence for app).

——————————————————————————————

## 5. Proposal Format and Due Date

(1) Cover letter — company, contact. (2) Confirmation scope is install only per reference docs. (3) Lump sum, payment terms, exclusions. (4) Availability/lead time. (5) Questions.

Submit to: [Insert contact name/email]

Due date: [Insert date]

This RFP is for installation labor only. Equipment and design are in the Site Camera folder reference documents (e.g. 3_Starlink_Security_Visual.html).