# Request for Proposal

# Single-Panel Solar Fence — Well Pump Area

Project: Single-panel solar fence (vertical PV mounting)

Location: Adjacent to well pump / 2000 gal water collection (5 HP pump) area

Delivery: Time and material (T&M) for this first installation; subsequent scope to be defined

Reference drawing set: 705 panel detail and associated structural/site drawings

——————————————————————————————

## 1. Scope of Work

The Owner requests proposals for the supply and installation of one (1) single-panel solar fence at the designated site. The fence consists of vertical support posts, concrete footings, and a single solar panel mounted in a vertical (portrait) orientation. The panel itself will be Owner-supplied; the Contractor shall furnish all structural materials, labor, and installation.

Required work includes:

Layout and excavation for two (2) concrete footings per structural drawing (each 18" × 36" plan, 3'-0" depth unless soils require otherwise).

Pouring concrete footings; Owner may perform excavation and concrete work by prior agreement—Contractor shall state in the proposal whether they will perform this work or assume Owner-furnished footings.

Supply and installation of vertical support structure (posts and horizontal framing) per structural drawing: Vertical posts 2×2 tube; Top horizontal 2×1 tube, 6" below top of post; Bottom horizontal 1×2 tube, 6" above top of footing; Clear panel opening height 8'-0".

Post-to-footing connection — Contractor shall choose one: Option A — Posts sunk/embedded in concrete; Option B — Base plate with bolt-hole connection (Owner to provide base-plate detail).

Panel attachment: Simple L-angle (e.g. 0.75" × 2.5") connected to Owner-supplied panel using factory mounting holes (M8, 5–Φ11). Panel set in opening; L-angle welded to fence post. Comply with EGING mounting requirements.

Orientation: Solar fence shall be oriented East–West. Layout verified in field.

Coordination with 2" PVC Schedule 40 Solar Conduit: Contractor shall state whether conduit to fence is in scope or excluded.

——————————————————————————————

## 2. Owner-Furnished Items

Solar panel(s): Owner will supply (reference EG-NT66-HU/BF-DG or equivalent per 705 panel detail).

Detail for post connection: Owner will provide for either sunk-post or base-plate, as selected by Contractor.

——————————————————————————————

## 3. Materials and Finish

Structural steel: Contractor shall specify type and grade. Owner does not prescribe.

Finish: No powder coat. Steel may weather to natural rust patina unless Contractor proposes alternative and Owner approves.

Hardware: M8 bolts and gaskets/washers per EGING. Contractor may specify galvanized or stainless.

——————————————————————————————

## 4. Contract Type and Pricing

This first installation is time and material (T&M). Proposal shall include: labor rate(s) and estimated hours; material cost; footing responsibility (Contractor vs Owner); conduit/electrical scope. Payment terms to be agreed. Subsequent panels may be contracted separately.

——————————————————————————————

## 5. Submittals and Coordination

Contractor confirms post-to-footing option and panel mounting (L-angle, M8, 5–Φ11). Deviations from structural drawing noted and approved by Owner.

——————————————————————————————

## 6. Drawing References

Single-panel solar fence isometric/elevation; Site plan; Structural (2×2 tube, 2×1, 1×2, 8'-0", 18"×36" footing, 3'-0" depth); Panel EG-NT66-HU/BF-DG; EGING 5.2.1 Bolts Mounting.

——————————————————————————————

## 7. Proposal Format and Due Date

Provide: (1) Cover letter, company name, contact, T&M confirmation; (2) Scope summary—footing, post option, conduit; (3) Pricing—rates, hours, materials, total T&M; (4) Schedule; (5) Exceptions or assumptions.

Proposal due: _________________________

Submit to: _________________________

This RFP covers one (1) single-panel solar fence, T&M. East–West. Owner-supplied panel. Sink or base plate; Owner supplies detail. L-angle to mounting holes, welded to post. No powder coat; Contractor specifies steel.