REQUEST FOR PROPOSAL

PLUMBER  —  Well Water System — Pressure Station at House

Rodriguez Residence — Maine Property

Owner / Developer:  RAC Development | Craig Engel

Date Issued:  March 05, 2026

Response Due:  ___________________________

Contact:  ___________________________

# Project Overview

RAC Development is soliciting bids for all plumbing work on the well water system at the Rodriguez residence in Maine. The plumber's scope is entirely at the house — picking up the supply line where it exits the ground and installing the complete pressure station. No underground or site work of any kind.

## Scope of Work — Supply Line Connection

Connect to 1-¼" water supply line stub-out at house foundation

Verify pipe material, transition fittings as needed

Install shutoff valve at entry point

## Scope of Work — Pressure Station Equipment

Install Y-strainer on inlet (20 mesh minimum)

Install Grundfos booster pump (confirm supply — owner or plumber)

Install check valve on booster pump outlet

Install Amtrol WX-302 (80 gal) pressure tank or approved equal

All fittings, unions, and isolation valves

## Scope of Work — Controls & House Distribution

Install pressure switch (40/60 PSI) — electrician wires it

Connect pressure station to house water distribution

Install pressure gauge

Pressure test entire system — document and sign off

## Scope Exclusions

No excavation, trenching, or any underground work

No electrical wiring — electrician wires booster motor and pressure switch

Nothing at wellhead, storage tank, or trench run

## Bid Requirements

Lump sum price — pressure station installation

Clarify if booster pump is supplied by plumber or owner

Estimated duration

List of similar rural/well water projects

Proof of insurance and plumbing license

Payment terms

# Questions & Submittals

Submit all questions and bids in writing to the contact above by the Response Due date.

Company Name:  ___________________________

Signature:  ___________________________

Date:  ___________________________

License #:  ___________________________

Lump Sum Bid Amount:  ___________________________