# Request for Proposal

# Well Pump Solar Power System — Electrical Installation Only (Solar Electrical Vault)

Project: Rodriguez Residence

Location: Oak Valley Estates Lot 5, Kanarraville, UT — Well vault and fence-mounted solar array

Delivery: Lump sum for electrical labor per attached scope

Reference: All equipment Owner-furnished. See Well Pump\HTML\4_Well_Solar_Visual.html and related docs in the Well Pump folder.

——————————————————————————————

## Definition — Solar Electrical Vault Scope

Vault: Underground enclosure, 36" x 42", hatch at grade. Contains Victron MPPT, 24V battery bank (100Ah x2 series), Victron MultiPlus-II 24/3000 2x120V. DC in from solar; AC out to VFD.

Already installed (do not touch): 705W solar panel (fence-mounted); Franklin pump, pump wire, VFD Commander Pro at wellhead.

Contractor electrical scope: (1) PV combiner at array (2) DC cable array to vault (3) Vault interior wiring (4) AC cable vault to VFD AC input only (5) Commissioning and labeling.

——————————————————————————————

## 1. Scope of Work — Electrical Installation Only

Proposals are for ELECTRICAL INSTALLATION ONLY. No structural, fence, or concrete. All equipment Owner-furnished. Contractor wires per reference.

Flow: 705W panel (already up) → PV Combiner at array → DC cable to vault → Victron MPPT → 24V battery bank → Victron MultiPlus-II 24/3000 → AC to VFD. VFD and pump already installed — AC input terminals only.

PV Combiner at array: Mount combiner (fuse, 40A DC breaker, SPD) near panel. MC4 in, DC out to vault. Bond ground to array and GEC.

DC cable: Run USE-2/PV in conduit from combiner to vault. Terminate at charge controller.

Vault: Mount MPPT; wire PV in, battery out. Place batteries (24V, 100Ah x2 series); fuse; connect to controller and inverter DC. Mount MultiPlus-II; DC from battery, AC L1/L2 to run to VFD. Sequence: batteries to controller and inverter first, then PV to controller. Never PV before batteries.

AC cable: Vault to VFD Commander Pro AC input only (L1, L2, ground). 240V. Do not open VFD.

Commissioning: Batteries on → controller PV reading → inverter on → confirm VFD power. Do not test pump. Document string voltage/current if requested. Label all.

——————————————————————————————

## 2. Owner-Furnished

705W panel (installed). Franklin pump and VFD (installed). Combiner, Victron MPPT, battery bank, MultiPlus-II 24/3000, cable, conduit, fuses, misc. per reference. Vault as existing.

——————————————————————————————

## 3. Contractor Provides

Labor: combiner mount, DC run, vault wiring per sequence, AC run to VFD, terminate at AC input only, commissioning, labeling. Tools and test equipment. Confirm end-to-end operation.

——————————————————————————————

## 4. Pricing

Lump sum for electrical install only. Payment terms. Exclusions or assumptions.

——————————————————————————————

## 5. Proposal Format and Due Date

(1) Cover letter — company, contact. (2) Confirmation scope is electrical only per reference. (3) Lump sum, payment terms, exclusions. (4) Availability/lead time. (5) Questions.

Submit to: [Insert contact name/email]

Due date: [Insert date]

This RFP is for electrical installation labor only. Equipment and design are in the Well Pump folder (e.g. 4_Well_Solar_Visual.html). Panel and VFD/pump already in place.