# REQUEST FOR PROPOSAL

Concrete Foundation & Flatwork

## PROJECT INFORMATION

Project: Rodriguez Residence - Oak Valley Estates Lot 5

Location: Liberty Lane, Kanarraville, UT 84742

Owner: ARJR Kanarraville Holdings, LLC

General Contractor: RAC Inc

Contact: Craig Engel, racinc19@gmail.com, (435) 229-7360

RFP Issue Date: February 4, 2026

Proposal Due Date: February 18, 2026

Anticipated Start: March 2026

## SCOPE OF WORK

Complete concrete foundation, suspended slab, and flatwork per structural plans Structural_Plan_set_2 (Sheets S1-S8). Includes all labor, materials, equipment, forming, reinforcing, pumping, and finishing. Site preparation by separate contractor (BlackstoneX LLC).

## FOUNDATION SCOPE - PER STRUCTURAL PLANS

| Item | Quantity | Unit | Unit Price |
| --- | --- | --- | --- |
| FOOTINGS & FOUNDATIONS |  |  |  |
| 18x12 strip footing - exterior | 402 | LF |  |
| 30x12 strip footing - exterior | 293 | LF |  |
| 18x12 strip footing - interior | 57 | LF |  |
| 30x12 strip footing - interior | 94 | LF |  |
| Spread footings - per plan schedule | 33 | EA |  |
| 30" stem wall | 406 | LF |  |
| 8" foundation wall x 13' high | 107 | LF |  |
| 8" foundation wall x 10' high | 236 | LF |  |
| SLABS |  |  |  |
| Basement floor slab w/ #3 rebar @ 24" o.c. | 2,181 | SF |  |
| Main floor slab w/ #3 rebar @ 24" o.c. | 2,600 | SF |  |
| Suspended 8" slab over 20ga B-deck w/ #4 rebar | 1,180 | SF |  |
| Garage stem walls | 72 | LF |  |
| Garage slab w/ #3 rebar @ 24" o.c. | 1,766 | SF |  |
| ADDITIONAL SCOPE |  |  |  |
| Concrete pumping | 5 | EA |  |
| Anchor bolts for holddowns | 10 | EA |  |
| TOTAL |  |  |  |

## SPECIFICATIONS

• All work per structural plans Structural_Plan_set_2 (Sheets S1-S8)

• Concrete: 3,500 PSI minimum for footings/walls, 4,000 PSI for slabs

• Rebar: ASTM A615 Grade 60, placed per structural details

• Foundation waterproofing: Dampproofing or membrane per code

• Perimeter drain: 4" perforated pipe in gravel with filter fabric

• Finish: Trowel finish for interior slabs, broom finish for garage

• Curing: 7 days minimum wet cure or approved curing compound

## EXCLUSIONS

• Site excavation and preparation (by BlackstoneX LLC)

• Foundation backfill (by BlackstoneX LLC)

• 20ga B-deck for suspended slab (by others)

• Building permits and inspection fees

• Exterior flatwork (driveways, sidewalks - separate bid)

## REQUIRED SUBMITTALS

• Itemized pricing per schedule above with unit costs

• Payment terms and schedule

• Project timeline with key milestones

• Warranty terms (minimum 1 year workmanship)

• Proof of insurance: $2M general liability, workers comp

• Contractor license number

## SUBMISSION

Submit proposals by February 18, 2026 to:

Craig Engel

RAC Inc

racinc19@gmail.com

(435) 229-7360

Questions due by February 11, 2026. Site visit available upon request.